MEMORANDUM

Date:		July 29, 2026

To:		TBD – Project Manager

From:		Jeffrey W. James – Environmental Manager

Copies:	Kevin Ingle, Amy Alfonso-Perez, Greg Bowne, Jon Sands, Brian Blair, Sara Stevenson, Megan Arp, Michael Sherron, Brent Setchell, Pam Barr, Nicole Monies, Nikki Gilmer, Kristin Caruso-Scalar, Brooke Feagle-Atkins, Leah Paulsen-Atkins, Catie Neal-KCA, Julia Holtzhausen-HNTB, Katherine Lawlor-HNTB, Brett Berube-RK&K, Lisa Brinson, Michael Adams-AIM, Robin Parrish, Nikesh Patel, Camden Mills, Alvimarie Corales, Corinne Arriaga, EMO file

Subject:	Financial Project ID:	TBD

Federal Project ID:	TBD

County/Section No:	Sarasota

Description:		Boulevard of the Arts Complete Streets

Environmental Certification for construction authorization

This is a class of action determination for the subject project.

This project is a Type 1 Categorical Exclusion under 23 CFR 771.117(c), effective October 28, 2018, for the following type of project action:

3. Construction of bicycle and pedestrian lanes, paths and facilities.

It has been determined that the project will not cause significant adverse impacts to local traffic patterns, property access, community cohesiveness, planned community growth, wetlands, land use patterns, air quality, noise, water quality, navigation, or National System of Wild and Scenic Rivers.  The project will not cause significant impacts to floodplains and will not have significant contamination involvement. Additional right-of-way will not be required, and there will be no relocations.

The project area was assessed for potential impacts to construction from contamination or hazardous materials. The contamination evaluation assessed 16 sites that occur within a 500-ft buffer of the project, which may act as potential contamination sources to the project. Of these 16 sites, two (2) were assigned a no risk rating, 13 were assigned a low risk rating, and one (1) was assigned a medium risk rating. For the sites rated as “no” or “low” risks, no further action is required. These locations have been determined not to have any contamination risk to the study area at this time. The site assigned a “medium” risk rating is D&R Automotive of Sarasota Inc. (formerly a retail gas station). While Level II testing is typically recommended for medium rated sites, in this case, it is anticipated that there may be potential unresolved contamination issues which could require additional effort relative to National Pollutant Discharge Elimination System (NPDES) permitting. Therefore, Level II field testing is not recommended at this time, but the site should be reviewed prior to construction to verify regulatory status. Additional information is available in the project Contamination Technical Memorandum. The Florida Department of Transportation (FDOT) District 1 Environmental Management Office (EMO) will also review the project for potential contamination involvement and provide any additional concerns during their review.

The project is located within the U.S. Fish and Wildlife Service Consultation Area for the piping plover, Florida scrub-jay, West Indian manatee, and Florida bonneted bat. The project is also located within the core foraging areas of four (4) wood stork colonies. It has been determined that the project will have no effect on federal endangered and threatened species or their critical habitats. The project occurs within highly urban, downtown Sarasota and does not have any involvement with suitable habitat for protected species. The project will not impact any wetlands or other surface waters, nor will it result in the removal of any native vegetation. The consultation keys for the manatee, bonneted bat, and wood stork which support no effect determinations for these species are attached. No further consultation is required for these species. Section 7-1.4: Compliance with Federal Endangered Species Act and other Wildlife Regulations of the Standard Specifications for Road and Bridge Construction manual will be adhered to for wildlife involvement during construction. If work is conducted outside of the existing right-of-way, or if staging areas are located outside of the existing right-of-way, please contact the Environmental Management Office.

Properties protected under Section 4(f) of the U.S. Department of Transportation Act are not acquired or used. However, the project does occur adjacent to two significant recreational resources, the Bay Recreational Area and Rosemary Park. It was found that there would be no use of either of these resources. The Office of Environmental Management (OEM) provided concurrence with these determinations on XXX XX, 2024. No construction staging may occur within these resources and these resources must remain open to the public throughout construction.

A Cultural Resource Assessment Survey (CRAS) was conducted for the project. The CRAS found that one previously recorded archaeological site occurs within the project’s Area of Potential Effects (APE). Site 8SO00041 (Tamiami Trail) is a pre-Contact shell midden with its northernmost portion recorded within the south ROW east of the 10th Street and US 41 roundabout. However, this site is presumed destroyed due to development of the project area. As a result of the historical/architectural field survey, five historic resources were documented within the APE, four buildings (sites 8SO02612, 8SO06831, 8SO06832, 8SO14598) and the Overtown Historic District (site 8SO03104). Overall, the newly identified and previously recorded historic buildings lack sufficient architectural features, and are not significant embodiments of a type, period, or method of construction. Thus, the resources do not appear individually eligible for listing in the National Register of Historic Places (NRHP); however, the Commercial style building at 1365 6th Street (8SO02612) appears to remain eligible as a contributing resource to the Overtown Historic District (8SO03104). The NR-listed Overtown Historic District is significant at the local level under Criteria A and C in the areas of Community Planning and Development, Ethnic Heritage/Black, and Architecture. The proposed work is limited to sidewalk widening, curb ramp warning detectors, and curb extensions at Central Avenue. Based on the proposed improvements, the project will not further alter the setting of the Overtown Historic District (8SO03104) in a negative way that will diminish or destroy the qualities and characteristics for which the resource is listed in the NRHP. As such, the proposed project will have no adverse effect to historic properties. The State Historic Preservation Officer (SHPO) provided concurrence with this finding on XXXX.

It was concluded that no significant impacts would result from this project.  No public hearing is required. However, a public meeting was held on May 23, 2024, to provide the public with a venue to provide feedback on the project.

The following is a list of any supporting activities (e.g., field reviews, as appropriate, etc.), reports, or technical studies that were prepared to support the determination reached on the checklist.

- ETDM Environmental Screening Tool (EST)

- Google Earth Streetview

- FEMA Floodplain Map review

- Cultural and Historic Desktop Analysis and coordination with SHPO

- Species Determination of Effect Keys

- Contamination Memorandum

- Section 4(f) Officials with Jurisdiction (OWJ) coordination and OEM coordination for “no use” determinations

Please note that projects which are determined to be Type 1 Categorical Exclusions are determined to have general Location and Design Concept Acceptance as described in 23 CFR 771.117(c).

This office requests notification if the scope of the project changes at a later date.  A copy of this Environmental Determination is available in the project files.

The environmental review, consultation, and other actions required by applicable Federal environmental laws for this project are being, or have been, carried out by the Florida Department of Transportation (FDOT) pursuant to 23 U.S.C. § 327 and a Memorandum of Understanding dated May 26, 2022, and executed by the Federal Highway Administration and FDOT.

JWJ/brb